PX-8 VFS - a virtual file system for the Epson PX-8.

Version 0.96 of 30 Jul 99.

Written by C W Rose.

INDEX.

1. OVERVIEW.

2. PX8VFS.EXE

3. PX8UTY.EXE

4. DISK FORMAT NOTES.

5. CABLE PINOUTS.

OVERVIEW.

The PX-8 is a small CP/M portable made by Epson and sold in the

US as the Epson Geneva.  The PX-8 itself has a 80x8 LCD screen,

but various OEM versions such as the PX-4 and HC-41 have a 40x8

screen.  As far as I can tell these OEM versions will run most

PX-8 software, including this file system, but I can't find

descriptions of all the machines and haven't done much testing.

The OS/2 version was compiled with IBM VAC++ 3.0 and tested under

OS/2 Warp Connect.  The MSDOS version was compiled with Microsoft C

version 7.0, and uses Marshallsoft's Personal Communications Library

version 6.0 for serial I/O.  It was tested under MSDOS 5.0.

In both cases, the program needs a 16550 UART to work correctly;

it will display a warning if a 16550 isn't found.

OS/2's multitasking gives better performance than the polling

needed for the MSDOS version; the VFS can be run quite easily

in the background in OS/2.   However, MSDOS performance is

acceptable.  The programs haven't been tested under any form

of Windows.

There are two programs in the suite, PX8VFS.EXE and PX8UTY.EXE.  The

first is the actual file system driver, which connects to the IBM PC

serial port, and the second is a utility to manipulate the files which

comprise PX-8 CP/M filesystems on an IBM PC.  PX8UTY.EXE can build

filesystems, check filesystems, show directories, erase files, and

move files between CP/M and MSDOS.

PX8VFS.EXE

px8vfs [-h] [-p port] [-r #] [-v] [-x #] [files...i]

-h		Help

-p port		Set communications port.

-r #		Set filesystems read only.

-s		Set comm port speed to 4800 baud.

-u #		Set unit number.

-v		Set verbose mode.

-x #		Set debugging level.

files...	PX-8 disk image filenames.

Overview.

On start up PX8VFS.EXE attempts to open up to four disk image files;

if sufficient names are not given on the command line, then the

default names PX8DSK.00[0123] are used.  If the -u option is set,

then only two disk image files will be opened.  If no disk image file

can be opened then the program exits.

The program then attempts to open a serial port at 38400 baud, 8n1.

The port used is either the one specified on the command line, or

if no port is given, COM2.  If a port cannot be opened then the

program exits.

Finally, if the debugging level is sufficiently high, the program

opens a log file.

The program then waits for messages from the serial port in EPSP

(Epson Serial Communication Protocol) format, and responds to them as

appropriate.  It exits if any key is pressed on the IBM PC keyboard.

Options.

The -h option gives a summary of the command line options.

The -p option sets the name of the serial port to be used.  The

program attempts to open the port using the -p option's argument

as the port name.  The default port on the IBM PC is COM2.

The -r option sets read-only access to the drives; its argument is

a bit map converted to decimal, with bit 1 representing drive 0 (d:)

and bit 4 representing drive 3 (g:).  If a bit is set, the drive is

read-only.  For example, the option to set drives d: and f: read-only

would be -r 5.

The -s option sets the port speed to 4800 baud.  This option was added

in case it was needed for low speed machines, but at present there seems

no way to configure the PX-8's serial port to anything other than 38400

baud when EPSP is in use.

The -u option sets the unit emulation number; its argument is either

1, for drives d: and e:, or 2, for drives f: and g:.  Normally the

program pretends to be two two-drive units, but if an actual one or

two-drive unit is present then only two more drives can be added,

since the PX-8 expects two drives per unit and cannot handle more

than four drives.

The -v option causes the program to write progress messages to

standard output.

The -x option sets the debug level.  The higher the level, the

more detailed the information written to stderr.

If the debug level is >= 12, the CP/M filesystem is not changed.

If the debug level is >= 9, sector reads and writes are reported.

If the debug level is >= 3, debugging information is written to

a log file.  The default logfile name is EPSP.LOG.

Bugs.

Most of the PX-8 software is fairly straightforward, and simply

reads and writes sectors to the CP/M disks, but COPYDISK, which

does disk formatting and fast copying, is slightly odd in its

behaviour.  Experiments with a TF-20 seem to show that COPYDISK

sends the EPSP disk copy command only when copying from drive

0 to drive 1 of a given unit.  When copying from drive 2 to

drive 1 (or between any drives on different units) the very much

slower sector copy is used.  This program matches COPYDISK, for

better or worse.

If a drive is set read-only then the program will certainly not

write to the drive, but the Epson PX-8 may get confused about

the drive state.  It returns an error "Bad sector" rather than

"Drive R/O" on the first attempt to write to a protected drive.

A subsequent write gives the correct error message.  This seems

to be a problem in the PX-8 BDOS, since PXVFS.EXE is sending the

specific EPSP message for a write-protect error.

Due to problems with the serial port library, the MSDOS version

relies on a timing loop for good performance.  If the PX-8 can

read from the file system but not write to it, try running the

software on a slower machine.

PX8UTY.EXE

px8uty -b|c|d|e|f|r|u|w disk [file]

-b	Build a new filesystem

-c	Check a filesystem.

-d	Show a CP/M directory.

-e	Erase a CP/M file.

-r	Read a CP/M file.

-u #	Set the CP/M user number.

-v	Set verbose mode.

-w	Write a CP/M file.

-x #	Set debugging level.

Overview.

This program combines a number of functions needed to manipulate

PX-8 format CP/M filesystems on an IBM PC.  It builds filesystems,

checks filesystems, displays filesystem directories, erases files,

and copies files between CP/M and MSDOS.

Options.

The -b option builds a new filesystem, filling it with Oxe5.

The -c option checks a filesystem for consistency; it reports on user

areas, filename characters, and multiply allocated data blocks.

The -d option shows a CP/M directory.

The -e option erases a CP/M file; note that the exact filename must

be given, to allow files with invalid names to be deleted if necessary.

The -r option reads a file from CP/M and copies it to MSDOS.

The -u option takes an argument between 0 and 31, and sets the user

area for the current command accordingly.  The default user area is 0,

except for the directory command, which shows all user areas.

The -v option sets verbose mode, which has no effect.

The -w option reads a file from MSDOS and copies it to CP/M.  Checks

are made to ensure that there is sufficient space available before

the file is copied.

The -x option sets the debug level.  The higher the level, the more

detailed the information that is produced by a given command.  Since

the actual output varies with the command being run, the effects are

summarised in the following table:

If the debug level is >= 12, the CP/M filesystem is not changed.

If the debug level is >= 9, data block reads and writes are reported.

If the debug level is >= 6, directory extent reads and writes are reported.

Option -b

If the debug level is >= 3, filesystems are built with some

directory entries in place.

If the debug level is >= 6, filesystems are built with additional

incorrect directory entries.

Option -c

If the debug level is >= 3, deleted files are included in checks.

Option -d

If the debug level is > 0, then extents are listed.

If the debug level is >= 3, then block numbers are listed.

If the debug level is >= 6, then deleted and empty files are listed.

DISK FORMAT NOTES.

A PF-10 drive has 40 tracks numbered 0 to 39.  Track 39 is not

used to maintain compatibility with the QX-10, and tracks 0 to

3 are reserved for the OS. There are 64 sectors of 128 bytes

per track, numbered 1 to 64, or 4 blocks, or 8K.

Blocks are 2K in size, and drives are 320K or 160 blocks.  The

first 16 blocks are reserved for the OS, so drive capacity is

288K or 144 blocks.

The directory takes up the first block, and thus contains 64

extents of 32 bytes; each extent can reference 16 blocks or 32K.

Since the full extent capacity of 256 sectors cannot be stored

in one byte, the extent count directory entry holds the current

half-extent; counts of 0 and 1 refer to the first extent of a

file, 2 and 3 to the second, and so on.

CABLE PINOUTS.

The Epson PX-8 cable #724 is used to connect a PX-8 to a modem,

and has the following pinouts:

Mini-DIN    DB-25

Shell - E   - 1

1 - GND - 7

2 - TxD - 2

3 - RxD - 3

4 - RTS - 4

5 - CTS - 5

6 - DSR - 6

7 - DTR - 20

8 - DCD - 8

The pin numbers on the Mini-DIN are:

1 2

3 4   5

6 7 8

Note the asymmetry of pins 3,4 and 5.

With the standard RS-232 D-25 pinouts of:

1 - Frame ground

2 - TxD

3 - RxD

4 - RTS

5 - CTS

6 - DSR

7 - Signal ground

8 - DCD

20 - DTR

the cable adapter that works in 95% of situations is:

DB-25      DB-25

1 <------> 1

2 <------> 3

3 <------> 2

4 <-+----> 8

5 <-+

6 <------> 20

7 <------> 7

8 <----+-> 4

+-> 5

20 <------> 8

Note that it is symmetrical.  It is also overkill, and disables all

hardware handshaking, but it works for both disk and RS-232 connections

when connected to an IBM PC.

END OF FILE.